# Assignment 2: Dining Philosophers

The goals for this assignment are as follows

- Familiarize yourself with the Python threading library.

  - Launching multiple threads

  - Sharing locks between threads

- Implement LED blinking capabilities

- Use button interrupts for killing threads

# Problem

Five philosophers are dining together at a table with five forks. Each philosopher shares their forks with neighboring philosophers and needs both forks (left and right) to eat. When a philosopher is done eating, it relinquishes the forks and takes a nap. Finally, when the philosopher is finished with the nap, it will wait, starving, for the two pairs of forks (left and right) to be freed to eat.

Thus, there are 3 possible states for each philosopher

- EATING: the philosopher has both forks (left and right)

- NAPPING: the philosopher is finished eating

- STARVING: the philosopher is waiting to have both forks (left and right)

# Part A2.1:

  - Write code for the dining philosophers problem. Use five LEDs, one for each philosopher, and five locks for forks. The five LEDs will be the four on-board green LEDs above the buttons and one of the on-board RGB LEDs that we saw in Lab1 (make it green to match the other LEDs).

  - Find appropriate durations for the philosophers to be eating and napping. Consider choices such that your threads do not go into a constant starvation. (i.e., should napping time be greater than or less than eating time?)

  - When one of the philosophers is eating, both forks are used by that philosopher, and the LED should blink at a higher rate to indicate “eating”.

  - When a philosopher is napping, the LED should blink at a lower rate to indicate “napping”.

  - When a philosopher is waiting for forks, its LED should be off to indicate “starving”.

  - The code must run forever. To terminate the program, you have to use push buttons.

# Part A2.2:

  - In this part, you use the random library to generate random numbers for the eating and napping states. By using random.randint(a, b) you can get a random number between a and b.

  - You have to set the boundaries for your random number (a, b) such that napping is not longer than eating, and therefore your threads do not go to a constant starvation.

# Deliverables

Each student must submit the following individually

- A PDF report detailing your workflow and the relevant Jupyter notebook cells relating to your progress.

  - Your report should detail your workflow throughout the assignment. Be sure to discuss any difficulties or troubles you encountered and your troubleshooting procedure. Also, detail your thought process which led to the design and implementation of the code. For

example, describe your top-down design methodology (i.e., how did you split the large task into smaller, more incremental jobs? How were you able to test each of these smaller parts?)

  - Please also include a brief discussion on the timings for EATING, NAPPING, and STARVING you chose to avoid a deadlock.

- Your complete Jupyter notebook, downloaded as a PDF attached at the very end of your report

  - You can do this by selecting ‘File -> Print Preview’, then printing to PDF from the browser.

  - Use a PDF stitching tool like pdfjoiner to join your Report and Jupyter Notebook into a single PDF file

- Video demonstration of your code working on the PYNQ board. Please limit videos to 60 seconds and upload them to a video-sharing site, and include the link in your PDF report.

Each team should submit the following: one per team

- All relevant code (.ipynb, .py, .cpp, .c, etc. files) pushed to your team’s git repo.